EARTHWORK AND UTILITIES BREAKDOWN

EARTHWORK

1. Install 770' of silt sock

2. Install a tracking pad

3. Saw cut asphalt

4. Removal of storm piping

5. Removal of concrete and asphalt

6. Saw cut & removal of new entry way

7. Inlet protection

8. Dig 4' walls & footings

9. Dig interior pads

10. Dig elevator pit

11. Backfill inside & outside walls that meet up to concrete or asphalt with compacted

granular material

12. Dig & backfill dumpster enclosure

13. Install 6" of compacted gravel under floor slab

14. Install 6" of compacted gravel under sidewalks

15. Cut & fill site to subgrade (Cut: 6,758yrds - Fill: 4yrds)

16. Install 12" of breaker run in parking lot

17. Install 18" of compacted gravel in parking lot

18. Haul in topsoil & spread 6"

UTILITIES

Sewer

1. Connect to existing stub out at property line

2. Install 20' for 8" pvc pipe

3. Install 8" building riser inside building

Water

1. Connect to existing stub out at property line

2. Install 20' of 8" c900 water line

3. Install ductile iron riser inside building

4. Perform pressure test and safe water sample

Storm

1. Install 364' of 12" hdpe pipe

2. Install 52' of 10" hdpe pipe

3. Install 10' of 11" pvc

4. Install 65' of 8" hdpe pipe

5. Install 61' of 6" hdpe pipe

6. Install 24' of 4" sch40 pvc

7. Install 1012' of 6" draintile with sock

8. Install 5 - 48" manholes with covers

9. Install 2 - 36" catch basin with covers

10. Install 8 yard drains

11. Install ADS under drain system

Additionally, the sponsor will be following the Remedial Action Plan (attached) which was submitted and approved by the WDNR (approval attached) specifically for this project. The amount for this work has been reflected in the attached budget for a total of $172,050 with $51,243 allocated to Atlas Environmental consultants and $120,807 allocated to Clean Vapors.